Devices detected are of the following classes:

CLASS_UNSPEC: used by the probe to say 'look for everything'

CLASS_NETWORK: network adapters

CLASS_SCSI: SCSI (and some RAID) adapters

CLASS_MOUSE: mice

CLASS_AUDIO: sound cards/chips

CLASS_CDROM: CD-ROMs, CD-Rs, CD-RW, DVD-ROMs, etc.

CLASS_MODEM: modems; also, PCI serial ports (FIXME)

CLASS_VIDEO: video cards

CLASS_TAPE: tape drives

CLASS_FLOPPY: IDE, SCSI, and USB floppy drives. Generic PC floppy drives

are *not* detected

CLASS_SCANNER: scanners

CLASS_HD: hard drives, other random storage media

CLASS_RAID: RAID adapters. Not really used by anything currently.

CLASS_PRINTER: printers

CLASS_CAPTURE: video capture boards (think bttv)

CLASS_KEYBOARD: keyboards (only returned on Sparc hardware)

CLASS_MONITOR: DDC monitors

CLASS_USB: USB controllers

CLASS_SOCKET: Cardbus controllers. Might be expanded to PCMCIA controllers

in the future

CLASS_OTHER: anything else

The 'bus'es that it finds things on are the following:

BUS_PCI - also sees any currently configured Cardbus stuff

BUS_SBUS - this is currently a misnomer. It applies to anything pulled from

the OpenPROM probe on Sparc

BUS_SERIAL

BUS_PSAUX - the PS/2 mouse controller

BUS_PARALLEL

BUS_SCSI - this includes ide-scsi and USB storage, if configured

BUS_IDE

BUS_KEYBOARD - this is where it looks for Sparc keyboard information

BUS_DDC - monitors

BUS_USB

BUS_ISAPNP

BUS_OTHER - a placeholder, mostly

BUS_UNSPEC - again, used by the probe to find anything

Fields written in /etc/sysconfig/hwconf that are common to all devices

(note: some may be NULL; if so, they are not written):

class:

The device class

bus:

The 'bus' the device was probed on

detached:

This is for devices that the user wants to leave as configured,

even though they may not be probed. Cardbus cards, if detected,

are automatically marked as this; a user may want to mark this

themselves for some USB devices.

driver:

(always non-NULL)

The driver to use for the card; it means different things for

different device classes. For mice, it's a mouseconfig type;

for video cards, it's a mapping in the Xconfigurator 'Cards'

database; for *most* other things, it's a kernel module.

There are three special values:

- unknown

This means that we don't know what driver to use

- ignore

This means that we don't care what driver to use. Generally

assigned to PCI bridges and the like. In the current

implementation, there really is no difference between

'ignore' and 'unknown'.

- disabled

This means that we'd normally assign a driver, but the

device appears to be disabled in the BIOS; this is mainly

set for PCI devices. This can occur if, for example,

the user sets 'PnP OS: Yes' in their BIOS and no interrupts

are assigned to any cards.

desc:

(always non-NULL)

Some generic description of the device

device:

For network cards, this is 'eth' to indicate ethernet, and

'tr' to indicate token ring. For other devices, it is usually

the device node associated with a device. For example, for an

IDE device, it would be along the lines of 'hda'. For a serial

device, it could be 'ttyS1'.

Devices have additional fields depending on the 'bus' that they are

associated with (again, some may be NULL):

BUS_DDC

id:

A DDC PnP id from the Xconfigurator MonitorsDB

horizSyncMin:

horizSyncMax:

vertRefreshMin:

vertRefreshMax:

The minimum and maximum horizontal sync and vertical refresh

rates.

modes:

A list of VESA compatible resolutions supported by the

video card/monitor combo.

mem:

The amount of memory detected on the attached video card.

Yes, this is technically in the wrong place.

BUS_IDE

physical:

logical:

The physical and logical geometry for the drive, as gleaned

from /proc/ide. Not used by anything.

BUS_ISAPNP

deviceId:

The ISAPnP logical device id

pdeviceId:

The parent card ISAPnP id

compat:

Any compatibility PnP id, if available

native:

'1' if a kernel driver with native ISAPnP support exists

for this device. Gleaned from /lib/modules/`uname -r`/modules.isapnpmap

active:

'1' if the device is currently activated

cardnum:

The index of the card that this logical device is part of.

logdev:

The logical device number for this logical device.

io:

irq:

dma:

mem:

Resources currently in use by this card.

BUS_KEYBOARD:

BUS_PSAUX:

(none)

BUS_PARALLEL:

pnpmodel:

pnpmfr:

pnpdesc:

The PnP model name, manufaturer, and description of this device.

pnpmodes:

The PnP supported 'modes' for this device.

pinfo:

X and Y resolution of the printer, whether or not it supports

color and raw ASCII output, and if it uses a uniprint driver.

(Note: information returned by the parallel probe is not currently used

by kudzu)

BUS_PCI:

vendorId:

deviceId:

The PCI vendor and device ids.

subVendorId:

subDeviceId:

The PCI subvendor and subdevice ids.

pciType:

'1' for a normal PCI device. '2' for a Cardbus device.

BUS_SBUS:

width:

height:

freq:

monitor:

Used for monitors detected via OpenPROM; it specifies

the resolution and frequency.

BUS_SCSI:

host:

channel:

id:

lun:

The host, channel, id, and lun of the device.

BUS_SERIAL:

pnpmfr:

pnpmodel:

pnpcompat:

pnpdesc:

The serial PnP manufacturer, model, compatibility string,

and description of the device.

BUS_USB:

usbclass:

usbsubclass:

usbprotocol:

The USB class, subclass, and protocol. Used to identify

the device type.

usbbus:

usblevel:

usbport:

productrevision:

Other USB data. Not currently used.

vendorId:

deviceId:

The vendor and device IDs of the USB device.